IRSIM-CAP(1)             USER COMMANDS               IRSIM-CAP(1)

NAME

irsim-cap

A version of the  event-driven  logic-level  simulator,

IRSIM-9.0,  with  extensions  for  improved capacitance

measurements.

SYNOPSIS

irsim-cap [-s] prm_file sim_file ... [+hist_file] [-cmd_file

...]

DESCRIPTION

IRSIM-CAP is an event-driven logic-level simulator  for  MOS

(both  N  and P) transistor circuits.  It is based on IRSIM-

9.0 with certain extensions to allow improved ease and accu-

racy  when  measuring switching capacitances.  This man page

describes only the differences between IRSIM-9.0 and  IRSIM-

CAP.   For a description of the basic IRSIM commands see the

"irsim(1)" man page.

ESTIMATING CAPS WITH IRSIM-9.0

IRSIM-9.0 itself has a number of commands to aid the user in

estimating switching capacitance.  They are described below:

caplogfile [[+]file]

This command turns on  the  feature  which  accumulates

transitions  on capacitive nodes.  It also starts a log

file which contains a list of  all  capacitive  transi-

tions with columns for:

1)   the time of the event,

2)   the name of the switching node,

3)   its old and new values,

4)   the rise time and delay of the event, and

5)   the physical capacitance being switched.

captrace [-]node/vector

This command determines which nodes will be watched for

capacitive  transitions.   For  example,  "captrace  *"

causes all nodes to be watched.

sumcap

This command sums the physical (not switching)  capaci-

tance of all nodes in the circuit.

logfile [[+]file]

This command starts a logfile of all simulation output,

which  upon conclusion of the simulation will contain a

summary of accumulated switching capacitances  for  all

nodes.  By column, this summary includes:

1)   the name of the node,

2)   the physical capacitance on that node,

3)   the number of transitions that node made,

4)   the transition-capacitance product (i.e.  the

Sun Release 4.1    Last change: December 1993                   1

IRSIM-CAP(1)             USER COMMANDS               IRSIM-CAP(1)

effective switching capacitance), and

5)   the fraction  of  the  total  chip  switching

capacitance that the node accounts for.

PROBLEMS WITH IRSIM-9.0

There are, however,  a number of  problems  with  estimating

switching capacitance using these commands:

1)   IRSIM-9.0 does not count transitions on primary inputs.

This can be a problem if you're characterizing a module

and want to include it's input gate capacitance in  the

results.

2)   IRSIM-9.0 is not particularly clever in how  it  counts

transitions.   {L,  X,  and  H} are all considered dif-

ferent states and a change between any of these is con-

sidered  a transition.  What we're really interested in

is the number of energy consuming (low to high) transi-

tions.   Now if there were only L and H states then for

long simulations the number of L->H  transitions  would

just  be the total number of transitions that IRSIM-9.0

gives you divided by two.

But the X state complicates matters  and  can  cause  a

serious  overestimate  of  switching  capacitance.  For

example, if a node passes through the X  state  on  its

rise  from  L  to  H (L->X->H), two transitions will be

counted when actually only one  charging  occurred.   A

similar problem occurs when a node temporarily glitches

(e.g. L->X->L).

Counting "transitions" also  presents  a  problem  when

trying  to  calculate  the energy drawn from the supply

over a short sequence of events.  For example, if  only

one  step  is simulated, during which a node discharges

from H->L, then even though no energy  was  drawn  from

the supply the "transition" count is non-zero.

3)   "sumcap" is not particularly useful since it  sums  the

physical  capacitance  of  all  nodes including Vdd and

GND, which don't even switch.  What we'd really like is

to sum the effective 0->1 switching capacitance.

4)   IRSIM-9.0 offers no easy way to calculate  the  capaci-

tance switched during individual events in a long simu-

lation sequence.  Instead, you must simulate each event

separately  and  output a different capacitance summary

into multiple log files.

5)   We'd like to be able to tell how much  energy  is  con-

sumed  by  each  of the different components of a chip.

This is very difficult since sim  files  are  flat  and

Sun Release 4.1    Last change: December 1993                   2

IRSIM-CAP(1)             USER COMMANDS               IRSIM-CAP(1)

it's   nearly   impossible  to  tell  what  capacitance

belonged to what cell originally.

ESTIMATING CAPS WITH IRSIM-CAP

IRSIM-CAP attempts to solve some of these  problems  through

several additional commands.  These commands are extensions,

however, and you should still be able to use  the  old  com-

mands  to  duplicate  your earlier results.  NOTE: issuing a

"caplogfile" command is no longer necessary to enable  tran-

sition counting.

inputcap [on | off]

This command enables you to include transitions on pri-

mary  inputs  in your capacitance estimations.  It does

not, however, consider changes in  Vdd  and  GND  since

these  only  switch  once during power up and including

this energy would swamp out the  nodal  transitions  in

which  you're  actually  interested.  Without any argu-

ments "inputcap" returns the  current  setting  of  the

flag.

sumeffcap [[+]file]

IRSIM-CAP keeps track of 0->1 transitions  as  well  as

"transitions".   It  also  employs  a slightly improved

accumulation scheme where L->H counts as 1  transition,

X->H  counts as 0.5 transitions, and L->X counts as 0.5

transitions.  "sumeffcap" outputs the  total  effective

capacitance  that  has  been  charged from 0->1 so far.

Output goes either to  the  screen  or  a  file  (+file

causes  the  result  to  be appended to the file).  The

energy drawn from the supply to accomplish this  charg-

ing is, of course, Ceff*Vdd^2.

zeropower

This  command  zeros  out  the  accumulated  transition

counts  for  all  nodes in the circuit.  In conjunction

with "sumeffcap", this allows you to easily output  the

capacitance  switched  during individual cycles of your

overall simulation.

fprint [+]file [text...]

Output the specified text  to  file.   If  no  text  is

specified  a  blank  line  is inserted.  The '+' prefix

specifies the append option rather than overwrite.

This command could be  used  in  conjunction  with  the

'sumeffcap  [+]file'  command  to  annotate  the output

file.

capreport [[+]file]

This command produces a report somewhat similar to that

discussed in "logfile" above.  I wrote it in an attempt

Sun Release 4.1    Last change: December 1993                   3

IRSIM-CAP(1)             USER COMMANDS               IRSIM-CAP(1)

to give the user the ability to figure out the  capaci-

tance  switched  within  individual modules in a larger

chip.

Since the sim files are flat, a single node  can  actu-

ally  represent capacitances from many different inter-

connected modules on the chip.   But  since  the  nodes

from the different modules have been collapsed into one

node in the sim file, there's  no  way  to  tell  which

cells  may have originally contributed to the collapsed

nodal capacitance or how much of  a  contribution  each

module may have made.

If IRSIM was at least aware of the  names  of  all  the

nodes  that  contributed  capacitance  to the collapsed

node, then at least some guess could be made as to  how

much each node contributed.  Fortunately, an alias file

(*.al) is generated along with each sim file.  By load-

ing this in with the sim file using:

irsim-cap my.prm my.sim my.al -my.cmds

IRSIM-CAP becomes aware of how  many  nodes  originally

contributed  to the capacitance of each collapsed node.

Furthermore, often by looking at the aliased node names

(e.g.  adder0/B[15]  or  adder0/add_cs_0_0/COUTINV) you

can tell to what cell  the  node  originally  belonged.

Then  as an approximation, we can split the lumped node

capacitance evenly among all contributing nodes  (since

there's  no  way to derive a more accurate distribution

from the sim file).

"capreport" facilitates this procedure.  For each  node

and each node alias, it produces an entry which states:

1)   the name of the node or alias,

2)   the number of equivalent  nodes  contributing

to the collapsed node,

3)   the portion of the physical node  capacitance

attributed to this particular component node,

4)   the number of 0->1 transition  on  the  node,

and

5)   the effective capacitance charged  from  0->1

for this component node.

The output of "capreport"  can  be  grep'd  through  to

split  the  capacitance  results  into  different  com-

ponents.  For example, here's a script I used to parti-

tion the results of an adder simulation among the vari-

ous bits:

#! /bin/csh -f

rm -f bit.energy

Sun Release 4.1    Last change: December 1993                   4

IRSIM-CAP(1)             USER COMMANDS               IRSIM-CAP(1)

foreach n (0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15)

echo "Extracting bit $n"

echo -n "bit$n- " >> bit.energy

egrep "[$n]|bit[^/]*_$n/" cap.report | \

chop -f5 | sum >> bit.energy

end

echo "Extracting non-bit nodes"

echo -n "Remainder- " >> bit.energy

egrep -v "[[0-9]*]|bit" cap.report | chop -f5 | \

sum >> bit.energy

CAVEATS

Neither IRSIM-9.0 nor IRSIM-CAP store transition counts dur-

ing  any  of  their  "dump" commands.  As a result you can't

start a simulation, stop and save it, and continue it  later

expecting   to   get  the  correct  accumulated  capacitance

results.  If memory usage is becoming  a  problem  you  can,

however,  use the "flush" command to erase the event history

up to the current simulation time.  This does not  adversely

affect transition counts for either IRSIM-9.0 or IRSIM-CAP.

ON-LINE HELP

Try "help [inputcap |  sumeffcap  |  zeropower  |  fprint  |

capreport]" from within IRSIM-CAP.

EXAMPLE COMMAND FILE

model linear

vector IN1 IN1[{15:0}]

vector IN2 IN2[{15:0}]

vector OUT OUT[{15:0}]

h Vdd

l GND

analyzer IN1 IN2

analyzer OUT

inputcap ON

captrace *

|

| Set initial inputs

|

set IN1 1010000100000010

set IN2 1100000110111101

s 100

|

| Make transition and output incremental capacitance

|

zeropower

Sun Release 4.1    Last change: December 1993                   5

IRSIM-CAP(1)             USER COMMANDS               IRSIM-CAP(1)

set IN1 1001111110111110

set IN2 0101000010011011

s 100

fprint     cap.summary EFFECTIVE INCREMENTAL CAPS:

fprint    +cap.summary

sumeffcap +cap.summary

|

| Make transition and append incremental capacitance

|

zeropower

set IN1 1011010110111010

set IN2 0101101010011111

s 100

sumeffcap +cap.summary

fprint    +cap.summary

fprint    +cap.summary END OF SIMULATION

|

| Output capacitances including info. about aliased nodes

|

capreport cap.report

exit

SEE ALSO

irsim(1)

presim(1) (now obsolete)

rsim(1)

IRSIM-analyzer(3)

sim(5)

netchange(5)

AUTHORS

irsim-9.0 is copyrighted (C) 1988, 1990 by Stanford University under the

following copyright notice:

*********************************************************************

* Copyright (C) 1988, 1990 Stanford University.                     *

* Permission to use, copy, modify, and distribute this              *

* software and its documentation for any purpose and without        *

* fee is hereby granted, provided that the above copyright          *

* notice appear in all copies.  Stanford University                 *

* makes no representations about the suitability of this            *

* software for any purpose.  It is provided "as is" without         *

* express or implied warranty.  Export of this software outside     *

* of the United States of America may require an export license.    *

*********************************************************************

Capacitance modifications for irsim-cap were performed by Paul Landman,

UC Berkeley, 1993, and they are also provided "as is" without express

or implied warranty.

Sun Release 4.1    Last change: December 1993                   6