IRSIM(1)

NAME

irsim - An event-driven logic-level simulator for MOS circuits

SYNOPSIS

irsim [-s] prm_file sim_file ... [+hist_file] [-cmd_file ...]

DESCRIPTION

IRSIM is an event-driven logic-level simulator for MOS (both

N and P) transistor circuits.  Two simulation models are

available:

switch

Each transistor is modeled as a voltage-controlled

switch.  Useful for initializing or determining the

functionality of the network.

linear

Each transistor is modeled as a resistor in series with

a voltage-controlled switch; each node has a capaci-

tance.  Node values and transition times are computed

from the resulting RC network, using Chorng-Yeoung

Chu's model.  Chris Terman's original model is not sup-

ported any more.

If the -s switch is specified, 2 or more transistors of the

same type connected in series, with no other connections to

their common source/drain will be stacked into a compound

transistor with multiple gates.

The prm_file is the electrical parameters file that config-

ure the devices to be simulated.  It defines the capacitance

of the various layers, transistor resistances, threshold

voltages, etc... (see presim(1)).

If prm_file does not specify an absolute path then IRSIM

will search for the prm_file as follows (in that order):

1) ./<prm_file> (in the current directory).

2) ~cad/lib/<prm_file>

3) ~/cad/lib/<prm_file>.prm

If ~cad/ does not exist, IRSIM will try to use directory

/projects/cad.  The default search directory (~cad) can be

overriden by setting the environment variable CAD_HOME to

the appropriate directory prior to running IRSIM (i.e.

setenv CAD_HOME /usr/beta/mycad).

IRSIM first processes the files named on the command line,

then (assuming the exit command has not been processed)

accepts commands from the user, executing each command

before reading the next.

File names NOT beginning with a '-' are assumed to be sim

files (see sim(5)), note that this version does not require

to run the sim files through presim.  These files are read

and added to the network database.  There is only a single

name space for nodes, so references to node "A" in different

network files all refer to the same node.  While this

feature allows one to modularize a large circuit into

several network files, care must be taken to ensure that no

unwanted node merges happen due to an unfortunate clash in

names.

File names prefaced with a '-' are assumed to be command

files: text files which contain command lines to be pro-

cessed in the normal fashion.  These files are processed

line by line; when an end-of-file is encountered, processing

continues with the next file. After all the command files

have been processed, and if an "exit" command has not ter-

minated the simulation run, IRSIM will accept further com-

mands from the user, prompting for each one like so:

irsim>

The hist_file is the name of a file created with the dumph

command (see below).  If it is present, IRSIM will initilize

the network to the state saved in that file.  This file is

different from the ones created with the ">" command since

it saves the state of every node for all times, including

any pending events.

This version supports changes to the network through the

update command.  Also, the capability to incrementally re-

simulate the network up to the current time is provided by

the isim command.

COMMAND SUMMARY

@ filename            take commands from command file

? wnode...            print info about node's source/drain connections

! wnode...            print info about node's gate connections

< filename            restore network state from file

> filename            write current network state to file

<< filename           same as "<" but restores inputs too

| comment...          comment line

activity from [to]    graph circuit activity in time interval

ana wnode...          display nodes in analyzer window

analyzer wnode...     display nodes in analyzer window

assert wnode [m] val  assert that wnode equals val

back [time]           move back to time

c [n]                 simulate for n clock cycles (default:1)

changes from [to]     print nodes that changed in time interval

clock [node [val]]    define value sequence for clock node

clear                 clear analyzer window (remove signals)

d [wnode]...          print display list or specified node(s)

debug [debug_level..] set debug level (default: off)

decay [n]             set charge decay time (0 => no decay)

display [arg]...      control what gets displayed when

dumph filename...     write net history to file

exit [status]         return to system

flush [time]          flush out history up to time (default:now)

h wnode...            make node logic high (1) input

has_coords            print YES if transistor coordinates are available

inputs                print current list of input nodes

ires [n]              set incremental resolution to n ns

isim [filename]       incrementally resimulate changes form filename

l wnode...            make node logic low (0) input

logfile [filename]    start/stop log file

model [name]          set simulation model to name

p                     step clock one simulation step (phase)

path wnode...         display critical path for last transition of a node

print comment...      print specified text

printp                print a list of all pending events

printx                print all undefined (X) nodes

q                     terminate input from current stream

R [n]                 simulate for n cycles (default:longest sequence)

readh filename        read history from filename

report[level]         set/reset reporting of decay events

s [n]                 simulate for n ns. (default: stepsize)

stepsize [n]          set simulation step size to n ns.

set vector value      assign value to vector

setlog[file|off]      log net changes to file (off -> no log)

setpath               set search path for cmd files

stats                 print event statistics

t [-]wnode...         start/stop tracing of specified nodes

tcap                  print list of shorted transistors

time [command]        print resource utilization summary

u wnode...            make node undefined (X) input

unitdelay [n]         force transitions to take n ns. (0 disables)

update filename       read net changes from file

V [node [value...]]   define sequence of inputs for a node

vector label node...  define bit vector

w [-]wnode...         add/delete nodes from display list

wnet [filename]       write network to file

x wnode...            remove node from input lists

Xdisplay[host:n]      set/show X display (for analyzer)

COMMAND DESCRIPTIONS

Commands have the following simple syntax:

cmd arg1 arg2 ... argn <newline>

where cmd specifies the command to be performed and the argi

are arguments to that command.  The arguments are separated

by spaces (or tabs) and the command is terminated by a <new-

line>.

If cmd is not one of the built-in commands documented below,

IRSIM appends ".cmd" to the command name and tries to open

that file as a command file (see "@" command).  Thus the

command "foo" has the same effect as "@ foo.cmd".

Notation:

...  indicates zero or more repetitions

[ ]  enclosed arguments are optional

node name of node or vector in network

wnode

name of node or vector in network, can include '*'

wildcard which matches any sequence of zero or more

characters.  The pair of characters '{' and '}' denote

iteration over the limits enclosed by it, for example:

name{1:10} will expand into name1, name2 ... name10. A

3rd optional argument sets the stride, for example:

name{1:10:2} will expand into name1, name3, ... name7,

name9.

| comment...

Lines beginning with vertical bar are treated as com-

ments and ignored -- useful for comments or temporarily

disabling certain commands in a command file.

Most commands take one or more node names as arguments.

Whenever a node name is acceptible in a command line, one

can also use the name of a bit vector.  In this case, the

command will be applied to each node of the vector (the "t"

and "d" treat vectors specially, see below).

vector label node...

Define a bit vector named "label" which includes the

specified nodes.  If you redefine a bit vector, any

special attributes of the old vector (e.g., being on

the display or trace list) are lost.  Wild cards are

not accepted in the list of node names since you would

have no control over the order in which matching nodes

would appear in the vector.

The simulator performs most commands silently.  To find out

what's happened you can use one of the following commands to

examine the state of the network and/or the simulator.

set vector value

Assign value to vector. For example, the following

sequence of commands:

vector BUS bit.1 bit.2 bit.3

set BUS 01x

The first command will define BUS to be a vector com-

posed of nodes bit.1, bit.2, and bit.3. The second com-

mand will assign the following values:

bit.1 = 0

bit.2 = 1

bit.3 = X

Value can be any sequence of [0,1,h,H,l,L,x,X], and

must be of the same length as the bit vector itself.

d [wnode]...

Display.  Without arguments displays the values all

nodes and bit vectors currently on the display list

(see w command).  With arguments, only displays the

nodes or bit vectors specified.  See also the "display"

command if you wish to have the display list printed

out automatically at the end of certain simulation com-

mands.

w [-]wnode...

Watch/unwatch one or more nodes.  Whenever a "d" com-

mand is given, each watched node will displayed like

so:

node1=0 node2=X ...

To remove a node from the watched list, preface its

name with a '-'.  If wnode is the name of a bit vector,

the values of the nodes which make up the vector will

be displayed as follows:

label=010100

where the first 0 is the value of first node in the

list, the first 1 the value of the second node, etc.

assert wnode [mask] value

Assert that the boolean value of the node or vector

wnode is value.  If the comparison fails, an error mes-

sage is printed.  If mask is given then only those bits

corresponding to zero bits in mask take part in the

comparison, any character other than 0 will skip that

bit.  The format of the error message is the following:

(tty, 3): assertion failed on 'name' 10X10 (1010X)

Where name is the name of the vector, followed by the

actual value and the expected value enclosed in

parenthesis.  If a mask is specified, then bits that

were not compared are printed as '-'.

ana wnode...

This is a shorthand for the analyzer command (described

below).

analyzer wnode...

Add the specified node(s)/vector(s) to the analyzer

display list (see irsim-analyzer(3) for a detailed

explanation).  If the analyzer window does not exist,

it will be created.  If no arguments are given and the

analyzer window already exists, nothing happens.

Xdisplay [host:display]

You must be able to connect to an X-server to start the

analyzer.  If you haven't set up the DISPLAY environ-

ment variable properly, the analyzer command may fail.

If this is the case you can use the Xdisplay command to

set it from within the simulator.  With no arguments,

the name of the current X-server will be printed.

clear

Removes all nodes and vectors from the analyzer window.

This command is most useful in command scripts for

switching between different signals being displayed on

the analyzer.

"?" and "!" allow the user to go both backwards and forwards

through the network.  This is a useful debugging aid.

? wnode...

Prints a synopsis of the named nodes including their

current values and the state of all transistors that

affect the value of these nodes.  This is the most

common way of wandering through the network in search

of what went wrong.

The output from the command ? out looks like

out=0 (vl=0.3 vh=0.8) (0.100 pf) is computed from:

n-channel phi2=0 out=0 in=0 [1.0e+04, 1.3e+04, 8.7e+03]

pulled down by (a=1 b=1)  [1.0e+04, 1.3e+04, 8.8e+03]

pulled up [4.0e+04, 7.4e+04, 4.0e+04]

The first line gives the node's name and current value,

its low and high logic thresholds, user-specifed low-

to-high and high-to-low propagation delays if present,

and its capacitance if nonzero.  Succeeding lines list

the transistor whose sources or drains connect to this

node: the transistor type ("pulled down" is an n-

channel transistor connected to gnd, "pulled up" is a

depletion pullup or p-channel transistor connected to

vdd), the values of the gate, source, and drain nodes,

and the modeling resistances.  Simple chains of

transistors with the same implant type are collapsed by

the -s option into a single transistor with a "com-

pound" gate; compound gates appear as a parenthesized

list of nodes (e.g., the pulldown shown above).  The

three resistance values -- static, dynamic high,

dynamic low -- are given in Kilo-ohms.

Finally, any pending events for a node are listed after

the electrical information.

! wnode...

For each node in the argument list, print a list of

transistors controlled by that node.

tcap

Prints a list of all transistors with their

source/drain shorted together or whose source/drain are

connected to the power supplies.  These transistors

will have no effect on the simulation other than their

gate capacitance load.  Although transistors connected

across the power supplies are real design errors, the

simulator does not complain about them.

Any node can be made an input -- the simulator will not

change an input node's value until it is released.  Usually

on specific nodes -- inputs to the circuit -- are manipu-

lated using the commands below, but you can fool with a sub-

circuit by forcing values on internal nodes just as easily.

h wnode...

Force each node on the argument list to be a high (1)

input.  Overrides previous input commands if necessary.

l wnode...

Like "h" except forces nodes to be a low (0) input.

u wnode...

Like "h" except forces nodes to be a undefined (X)

input.

x wnode...

Removes nodes from whatever input list they happen to

be on.  The next simulation step will determine the

correct node value from the surrounding circuit.  This

is the default state of most nodes.  Note that this

does not force nodes to have an "X" value -- it simply

removes them from the input lists.

inputs

prints the high, low, and undefined input lists.

It is possible to define a sequence of values for a node,

and then cycle the circuit as many times as necessary to

input each value and simulate the network.  A similar

mechanism is used to define the sequence of values each

clock node goes through during a single cycle.

Each value is a list of characters (with no intervening

blanks) chosen from the following:

1, h, H     logic high (1)

0, l, L     logic low (0)

u, U        undefined (X)

x, X        remove node from input lists

Presumably the length of the character list is the same as

the size of the node/vector to which it will be assigned.

Blanks (spaces and tabs) are used to separate values in a

sequence.  The sequence is used one value at a time, left to

right.  If more values are needed than supplied by the

sequence, IRSIM just restarts the sequence again.

V [node [value...]]

Define a vector of inputs for a node.  After each cycle

of an "R" command, the node is set to the next value

specified in the sequence.

With no arguments, clears all input sequences (does not

affect clock sequences however).  With one argument,

"node", clears any input sequences for that

node/vector.

clock [node [value...]]

Define a phase of the clock.  Each cycle, each node

specified by a clock command must run through its

respective values.  For example,

clock phi1 1 0 0 0

clock phi2 0 0 1 0

defines a simple 4-phase clock using nodes phi1 and

phi2. Alternatively one could have issued the following

commands:

vector clk phi1 phi2

clock clk 10 00 01 00

With no arguments, clears all clock sequences.  With

one argument, "node", clears any clock sequences for

that node/vector.

After input values have been established, their effect can

be propagated through the network with the following com-

mands.  The basic simulated time unit is 0.1ns; all event

times are quantized into basic time units.  A simulation

step continues until stepsize ns. have elapsed, and any

events scheduled for that interval are processed.  It is

possible to build circuits which oscillate -- if the period

of oscillation is zero, the simulation command will not

return.  If this seems to be the case, you can hit <ctrl-C>

to return to the command interpreter.  Note that if you do

this while input is being taken from a file, the simulator

will bring you to the top level interpreter, aborting all

pending input from any command files.

When using the linear model (see the "model" command) tran-

sition times are estimated using an RC time constant calcu-

lated from the surrounding circuit.  When using the switch

model, transitions are scheduled with unit delay.  These

calculations can be overridden for a node by setting its

tplh and tphl parameters which will then be used to deter-

mine the time for a transition.

s [n]

Simulation step.  Propogates new values for the inputs

through the network, returns when n (default: stepsize)

ns. have passed.  If n is specified, it will tem-

porarily override the stepsize value.  Unlike previous

versions, this value is NOT remembered as the default

value for the stepsize parameter.  If the display mode

is "automatic", the current display list is printed out

on the completion of this command (see "display" com-

mand).

c [n]

Cycle n times (default: 1) through the clock, as

defined by the "clock" command.  Each phase of the

clock lasts stepsize ns.  If the display mode is

"automatic", the current display list is printed out on

the completion of this command (see "display" command).

p    Step the clock through one phase (or simulation step).

For example, if the clock is defined as above

clock phi1   1 0 0 0

clock phi2   0 0 1 0

then "p" will set phi1 to 1 and phi2 to 0, and then

propagate the effects for one simulation step.  The

next time "p" is issued, phi1 and phi2 will both be set

to 0, and the effects propagated, and so on.  If the

"c" command is issued after "p" has been used, the

effect will be to step through the next 4 phases from

where the "p" command left off.

R [n]

Run the simulator through n cycles (see the "c" com-

mand).  If n is not present make the run as long as the

longest sequence.  If display mode is automatic (see

"display" command) the display is printed at the end of

each cycle.  Each "R" command starts over at the begin-

ning of the sequence defined for each node.

back time

Move back to the specified time.  This command restores

circuit state as of "time", effectively undoing any

changes in between.  Note that you can not move past

any previously flushed out history (see flush command

below) as the history mechanism is used to restore the

network state.  This command can be useful to undo a

mistake in the input vectors or to re-simulate the cir-

cuit with a different debug level.

path wnode...

display critical path(s) for last transition of the

specified node(s).  The critical path transistions are

reported using the following format:

node -> value @ time (delta)

where node is the name of the node, value is the value

to which the node transitioned, time is the time at

which the transistion occurred, and delta is the delay

through the node since the last transition.  For exam-

ple:

critical path for last transition of Hit_v1:

phi1-> 1 @ 2900.0ns , node was an input

PC_driver-> 0 @ 2900.4ns    (0.4ns)

PC_b_q1-> 1 @ 2904.0ns    (3.6ns)

tagDone_b_v1-> 0 @ 2912.8ns    (8.8ns)

tagDone1_v1-> 1 @ 2915.3ns    (2.5ns)

tagDone1_b_v1-> 0 @ 2916.0ns    (0.7ns)

tagDone_v1-> 1 @ 2918.4ns    (2.4ns)

tagCmp_b_v1-> 0 @ 2922.1ns    (3.7ns)

tagCmp_v1-> 1 @ 2923.0ns    (0.9ns)

Vbit_b_v1-> 0 @ 2923.2ns    (0.2ns)

Hit_v1-> 1 @ 2923.5ns    (0.3ns)

activity from_time [to_time]

print histogram showing amount of circuit activity in

the specified time inteval.  Actually only shows number

of nodes which had their most recent transition in the

interval.

changes from_time [to_time]

print list of nodes which last changed value in the

specified time interval.

printp

print list of all pending events sorted in time.  The

node associated with each event and the scheduled time

is printed.

printx

print a list of all nodes with undefined (X) values.

Using the trace command, it is possible to get more detail

about what's happening to a particular node.  Much of what

is said below is described in much more detail in "Logic-

level Simulation for VLSI Circuits" by Chris Terman, avail-

able from Kluwer Academic Press.  When a node is traced, the

simulator reports each change in the node's value:

[event #100] node out.1: 0 -> 1 @ 407.6ns

The event index is incremented for each event that is pro-

cessed.  The transition is reported as

old value -> new value @ report time

Note that since the time the event is processed may differ

from the event's report time, the report time for successive

events may not be strictly increasing.

Depending on the debug level (see the "debug" command) each

calculation of a traced node's value is reported:

[event #99] node clk: 0 -> 1 @ 400.2ns

final_value( Load )  V=[0.00, 0.04]  => 0

..compute_tau( Load )

{Rmin=2.2K  Rdom=2.2K  Rmax=2.2K}  {Ca=0.06  Cd=0.17}

tauA=0.1  tauD=0.4 ns

[event #99: clk->1] transition for Load: 1 -> 0 (tau=0.5ns, delay=0.6ns)

In this example, a calculation for node Load is reported.

The calculation was caused by event 99 in which node clk

went to 1.  When using the linear model (as in this example)

the report shows

current value -> final value

The second line displays information regarding the final

value (or dc) analysis for node "Load"; the minimun and max-

imum voltages as well as the final logical value (0 in this

case).

The next three lines display timing analysis information

used to estimate the delays.  The meaning of the variables

displayed can be found Chu's thesis: "Improved Models for

Switch-Level Simulation".

When the final value is reported as "D", the node is not

connected to an input and may be scheduled to decay from its

current value to X at some later time (see the "decay" com-

mand).

"tau" is the calculated transition time constant, "delta" is

when any consequences of the event will be computed; the

difference in the two times is how IRSIM accounts for the

shape of the transition waveform on subsequent stages (see

reference given above for more details).  The middle lines

of the report indicate the Thevenin and capacitance parame-

ters of the surrounding networks, i.e., the parameters on

which the transition calculations are based.

debug [ev dc tau taup tw spk][off][all]

Set debugging level.  Useful for debugging simulator

and/or circuit at various levels of the computation.

The meaning of the various debug levels is as follows:

ev      display event enqueueing and dequeueing.

dc      display dc calculation information.

tau     display time constant (timing) calculation.

taup    display second time constant (timing) calculation.

tw      display network parameters for each stage of

the tree walk, this applies to dc, tau, and

taup.  This level of debugging detail is usu-

ally needed only when debugging the simulator.

spk     displays spike analysis information.

all     This is a shorthand for specifying all of the

above.

off     This turns off all debugging information.

If a debug switch is on then during a simulation step,

each time a watched node is encounted in some event,

that fact is indicated to the user along with some

event info.  If a node keeps appearing in this prinout,

chances are that its value is oscillating.  Vice versa,

if your circuit never settles (ie., it oscillates) ,

you can use the "debug" and "t" commands to find the

node(s) that are causing the problem.

Without any arguments, the debug command prints the

current debug level.

t [-]wnode...

set trace flag for node.  Enables the various printouts

described above.  Prefacing the node name with '-'

clear its trace flag.  If "wnode" is the name of a vec-

tor, whenever any node of that vector changes value,

the current time and the values of all traced vectors

is printed.  This feature is useful for watching the

relative arrival times of values at nodes in an output

vector.

System interface commands:

> filename

Write current state of each node into specified file.

Useful for making a breakpoint in your simulation run.

Only stores values so isn't really useful to "dump" a

run for later use, i.e., the current input lists, pend-

ing events, etc. are NOT saved in the state file.

< filename

Read from specified file, reinitializing the value of

each node as directed.  Note that network must already

exist and be identical to the network used to create

the dump file with the ">" command.  These state saving

commands are really provided so that complicated ini-

tializing sequences need only be simulated once.

<< filename

Same as "<" command, except that this command will

restore the input status of the nodes as well.  It does

not, however, restore pending events.

dumph [filename]

Write the history of the simulation to the specified

file, that is; all transistions since time = 0.  The

resulting file is a machine-independent binary file,

and contains all the required information to continue

simulation at the time the dump takes place.  If the

filename isn't specified, it will be constructed by

taking the name of the sim_file (from the command line)

and appending ".hist" to it.

readh filename

Read the specified history-dump file into the current

network.  This command will restore the state of the

circuit to that of the dump file, overwriting the

current state.

flush [time]

If memory consumption due to history maintanance

becomes prohibitive, this command can be used to free

the memory consumed by the history up to the time

specified.  With no arguments, all history up to the

current point in the simulation is freed.  Flushing out

the history may invalidate an incremental simulation

and the portions flushed will no longer appear in the

analyzer window.

setpath [path...]

Set the search-path for command files.  Path should be

a sequence of directories to be searched for ".cmd"

files, "." meaning the current directory.  For eaxmple:

setpath . /usr/me/rsim/cmds /cad/lib/cmds

With no arguments, it will print the current search-

path.  Initially this is just ".".

print text...

Simply prints the text on the user's console.  Useful

for keeping user posted of progress through a long com-

mand file.

logfile [filename]

Create a logfile with the specified name, closing

current log file if any; if no argument, just close

current logfile.  All output which appears on user's

console will also be placed in the logfile.  Output to

the logfile is cleverly formatted so that logfiles

themselves can serve as command files.

setlog [filename | off]

Record all net changes, as well as resulting error mes-

sages, to the specified file (see "update" command).

Net changes are always appended to the log-file,

preceding each sequence of changes by the current date.

If the argument is off then net-changes will not be

logged.  With no arguments, the name of the current

log-file is printed.

The default is to always record net changes; if no

filename is specified (using the "setlog" command) the

default filename irsim_changes.log will be used.  The

log-files are formatted so that log-files may them-

selves be used as net-change files.

wnet [filename]

Write the current network to the specified file.  If

the filename isn't specified, it will be constructed by

taking the name of the sim_file (from the command line)

and appending ".inet" to it.  The resulting file can be

used in a future simulation run, as if it were a sim

file.  The file produced is a machine independent

binary file, which is typically about 1/3 the size of

the sim file and about 8 times faster to load.

time [command]

With no argument, a summary of time used by the simula-

tor is printed.  If arguments are given the specified

command is timed and a time summary is printed when the

command completes.  The format of the time summary is

Uu Ss E P% M, where:

U => User time in seconds

S => System time in seconds

E => Elapsed time, minutes:seconds

P => Percentage of CPU time (((U + S)/E) * 100)

M => Median text, data, and stack size use

q    Terminate current input stream.  If this is typed at

top level, the simulator will exit back to the system;

otherwise, input reverts to the previous input stream.

exit [n]

Exit to system, n is the reported status (default: 0).

Simulator parameters are set with the following commands.

With no arguments, each of the commands simply prints the

current value of the parameter.

decay [n]

Set decay parameter to n ns. (default: 0).  If non-

zero, it tells the number of ns. it takes for charge on

a node to decay to X.  A value of 0 implies no decay at

all.  You cannot specify this parameters separately for

each node, but this turns out not to be a problem.  See

"report" command.

display [-][cmdfile][automatic]

set/reset the display modes, which are

cmdfile     commands executed from command files are

displayed to user before executing.  The

default is cmdfile = OFF.

automatic   print out current display list (see "d"

command) after completion of "s" or "c"

command.  The default is automatic = ON.

Prefacing the previous commands with a "-" turns off

that display option.

model [name]

Set simulation model to one of the following:

switch

Model transistors as voltage controlled switches.

This model uses interval logic levels, without

accounting for transistor resistances, so circuits

with fighting transistors may not be accuratelly

modelled.  Delays may not reflect the true speed of

the circuit as well.

linear

Model transistors as a resistor in series with a

voltage controlled switch.  This model uses a

single-time-constant computed from the resulting RC

network and uses a two-time-constant model to analyze

charge sharing and spikes.

The default is the linear model.  You can change the

simulation model at any time -- even with events pend-

ing -- as only new calculations are affected.  Without

arguments, this command prints the current model name.

report [level]

When level is nonzero, report all nodes which are set

to X because of charge decay, regardless on whether

they are being traced.  Setting level to zero disables

reporting, but not the decay itself (see "decay" com-

mand).

stepsize [n]

Specify duration of simulation step or clock phase. n

is specified in ns. (nanoseconds).  Floating point

numbers with up to 1 digit past the decimal point are

allowed.  Further decimals are trucated (i.e. 10.299 ==

10.2).

unitdelay [n]

When nonzero, force all transitions to take n ns.  Set-

ting the parameter to zero disables this feature.  The

resolution is the same as for the "stepsize" command.

stats

Print event statitistics, as follows:

changes = 26077

punts (cns) = 208 (34)

punts = 0.79%, cons_punted = 16.35%

nevents = 28012; evaluations = 27972

Where changes is the total number of transistions

recorded, punts is the number of punted events, (cns)

is the number of consecutive punted events (a punted

event that punted another event).  The penultimate line

shows the percentage of punted events with respect to

the total number of events, and the percentage of con-

secutive punted events with respect to the number of

punted events.  The last line shows the total number of

events (nevents) and the number of net evaluations.

Incremental simulation commands:

Irsim supports incremental changes to the network and

resimulation of the resulting network.  This is done incre-

mentally so that only the nodes affected by the changes,

either directly or indirectly, are re-evaluated.

update filename

Read net-change tokens from the specified file.  The

following net-change commands are available:

add     type gate source drain length width [area]

delete  type gate source drain length width [area]

move    type gate source drain length width [area] g s d

cap     node value

N       node metal-area poly-area diff-area diff-perim

M       node M2A M2P MA MP PA PP DA DP PDA PDP

thresh  node low high

Delay   node tplh tphl

For a detailed dscription of this file see netchange(5).

Note that this is an experimental interface and is

likely to change in the future.

Note that this command doesn't resimulate the circuit

so that it may leave the network in an inconsistent

state.  Usually this command will be followed by an

isim command (see below), if that is not the case then

it's up to the user to initilize the state of the cir-

cuit.  This command exists only for historical reasons

and will probably disappear in the future.  It's use is

discouraged.

isim [filename]

Read net-change tokens from the specified file (see

netchange(5)) and incrementally resimulate the circuit

up to the current simulation time (not supported yet).

ires n

The incremental algorithm keeps track of nodes deviat-

ing from their past behavior as recorded in the network

history.  During resimulation, a node is considered to

deviate from its history if it's new state is found to

be different within n ns of its previous state.  This

command allows for changing the incremental resolution.

With no arguments, it will print the current resolu-

tion.  The default resolution is 0 ns.

SEE ALSO

presim(1) (now obsolete)

rsim(1)

irsim-analyzer(3)

sim(5)

netchange(5)