.he               Documentation - how to use COMPS19

When a customer submits a ROM code pattern to Motorola to be

mask-programmed into a microcontroller, the factory generates a

listing of the code an returns this to the customer to verify

that it matches the original pattern.  The problem then arises of

how to verify this code against the original, quickly and

reliably.

Because of the reformatting done at the factory, a straight

text compare of the two files will not work.  One alternative is

to compare the code by hand, but this is extremely time-consuming

and error-prone.

There is one further complication involved in performing a

comparision.  The object code returned from the factory contains

not only the customer's code, but also (depending on the

particular device) may contain sections of self-test code or

other information which is automatically inserted by the factory.

Thus the comaprision must be performed only on ranges of memory

addresses which contain user code.  For these reasons COMPS19 was

developed to automate the comparision.

COMPS19 reads both the customer file(s) and the factory

verification file(s), converts them to binary data, compares each

one and reports any differences found.  The comparision is

performed according to information stored in a parameter file,

which defines the name of each valid block of memory addresses,

the start and stop address for each block, and ancilliary

information such as the device part number and memory buffer size

needed for the comparision.  A seperate parameter file is

required for each Motorola Microcontroller, and several are

included on this disk.

The command line format for COMPS19 is as follows:

C>COMPS19 prmfile custfile [custfile] -v verfile [verfile]

where     "prmfile" is the parameter file for the device

"custfile" is the name of the customer's object code

file (Motorola S-record format)

note that more than one file may be specified

"-v" indicates that the following filenames are the

factory verification files

"verfile" is the name of the verification file returned

from the RVU center (again more than one file may be

specified)

A minimum of four parameters must be specified on the

command line - parameter file name, customer file name, "-v", and

verification file name.

The screen display should look something like that shown

below:

MC68HC04P4_Code_ROM               start = 0000 end = 0fff

===========================================================

Code                              start = 0160 end = 0ff7

Vectors                           start = 0ffc end = 0fff

===========================================================

Reading customer file nn2v01b.s19

Reading verify file z401008p.ver

Checking Code range

Mismatch at 0160 length 0020

Checking Vectors range

As shown, the program lists the address ranges which are

being checked, and reports the starting address and length of any

mismatch which occurs.

For any microcontroller which has multiple address spaces, a

separate parameter file must be created for each address space,

and a seperate verfication run must be done for each space.  In

the example above, the code space for the 68HC04 was checked.  A

seperate run must be done for the data space as shown below:

MC68HC04P4_Data_ROM               start = 0000 end = 00ff

===========================================================

Data                              start = 0018 end = 005f

===========================================================

Reading customer file nn2v01b.s19

Reading verify file z401008d.ver

Checking Data range

Mismatch at 005a length 0006

PARAMETER FILES

===============

The following parameter files are included with the program:

File Name      Device         Comment

=========      ======         =======

HC04P4P        MC68HC04P4     CODE SPACE ONLY

HC04P4D        MC68HC04P4     DATA SPACE ONLY

68HC11A8       MC68HC11A8

68HC05C4       MC68HC05C4

68HC05B6       MC68HC05B6

Parameter files consist of ASCII text only, and are simple

to create for new devices.  They may be created using any text

editor which does not insert special formatting characters in the

file.  Wordstar on the IBM PC may be used in non-document mode,

and many other text editors also support "text only" disk files.

The format of the parameter file is shown below:

DEVICE_NAME    MAP_OFFSET     MAP_SIZE

RANGE          RANGE_START    RANGE_END

RANGE          RANGE_START    RANGE_END

.              .              .

.              .              .

.              .              .

The first line contains, from left to right, the part number

of the microcontroller, the starting location of the memory map,

and the size of the memory map.  Each of these parameters must be

seperated by "white space" (a tab or space character), and there

may be no spaces embedded in the name.  The following lines

define the name, start address, and end address of each block to

be compared.  Again, white space seperates each parameter, and no

embedded spaces may be used.  Let's look at each of these in turn.

The DEVICE_NAME is used only for information, to be

displayed on the screen during operation to confirm to the

operator that he has the correct parameter file.

DEVICE_NAME may be up to 33 characters long.

MAP_OFFSET specifies the first address to be loaded when

performing the comparision.  This is a hex number of one to

four characters with leading zeroes assumed and NO leading

dollar sign.  For most microcontrollers such as the 6804 and

6805 families, this may be set to 0.  MAP_OFFSET is used to

reduce the memory requrements in cases where the MCU's ROM

size is much smaller than the total map size, as in the case

of the MC68HC11A8.  For this device, set the MAP_OFFSET to

E000.

MAP_SIZE specifies the size of the memory block to compare.

This is also a hex number of 1 to 4 characters, leading

zeroes assumed.  In most cases this would be the size of the

MCU's address space.  In cases where a non-zero MAP_OFFSET

is used, subtract that offset from the MCU's map size to

obtain the MAP_SIZE.  For the 68HC11A8 example above,

MAP_SIZE should be 2000.

All lines after the first line specify memory blocks or

ranges which are valid user-defined ROM areas and must be

compared.  RANGE gives the name of the block (eg, "Vectors"

or "Data_ROM"), and may be up to 33 characters long.

RANGE_START gives the first valid address of that block, and

RANGE_END gives the last address of the block.  A maximum of

8 ranges may be defined in a parameter file.

The best way to explain it is to show an example.  Here is

the parameter file HC04P4P, defining the program ROM space for

the 68HC04P4.

MC68HC04P4_Code_ROM	0	1000

Code			160	FF7

Vectors			ffc	fff

Following the rules laid down above, the DEVICE_NAME is

"MC68HC04P4_Code_ROM", and the map is 4096 bytes long with no

offset (ie, an offset of zero).  There are two valid blocks in

the code space, "Code" which starts at $160 and ends at $FF7, and

"Vectors" which occupies the top 4 bytes of ROM ($FFC - $FFF).

Here is the parameter file "HC04P4D" which defines the Data

space in the 68HC04P4.

MC68HC04P4_Data_ROM	0	100

Data			18	5F

In this case, the map is $100 bytes long.  There is only one

block, the user data ROM starting at $18 and ending at $5F.

To create a new parameter file for a new device, simply

write down the user-accessible ROM areas as defined in the data

sheet for the device, then fire up your text editor and type it

in as shown above.